Censtar Small Display (Below 100,000/= LKR Total) Capture

Connector PIN Maping

10 PIN IDC Connector

| PIN | Description |
| --- | --- |
| 1 | NOT Connected |
| 2 | NOT USED |
| 3 | DATA |
| 4 | NOT USED |
| 5 | STROBE/ DATA LATCH |
| 6 | CLK |
| 7 | GND |
| 8 | NOT Connected |
| 9 | VCC (5V) |
| 10 | NOT Connected |

Captured Data

Captured files (here)

Decoding

Data is captured when the display is set to

5860.83/ 16.463/356.00 -> total / liters / fuel unit price

Total			- 5860.83

Liters 			- 16.463

Unit Fuel Price 	- 356.00

G, H, I (4-22) Cells shows the consecutive data from the display. All as similar because display is fixed.

One Byte in old code is now 12 bits.

Every 12 bits there is a strobe

12 bit should taken into 3 times 4bit values.

Center 4 bit is the index which is goes from 0 to F2

And the decoding is as below.

Display data behaviour

| Status | Display Data Sending (Yes/ No) | Continuous or Once |
| --- | --- | --- |
| Nozzle - In the Pump Value - Non Zero | No | No |
| Nozzle - Remove Value - Zero | Yes | Once |
| Nozzle - Removed Value - Non Zero | Yes | Continuous |

Total Liters

Example is below

The value it 626125.938 Liters.

LL               =  4,5 bytes index of Total price segmenst

Total Liters = Value in liters + (value in total * 10000000)