WAYNE 6 Digits Display Capture

Display has 3 rows,

Row1 - 6digits

Row2 - 6digits

Row3 - 4digits (Unit Price)

Totalizer

Connector PIN Maping

10 PIN IDC Connector

| PIN | Description |
| --- | --- |
| 1 | NOT USED |
| 2 | DATA 1 |
| 3 | DATA 2 |
| 4 | CLOCK |
| 5 | NOT USED |
| 6 | NOT USED |
| 7 | 7.86V |
| 8 | GND |
| 9 | 5V |
| 10 | GND |

Captured Data

Data points are received in every Second

One data point has 51 bytes of data

Clock is 50KHz

Decoding

There is two data lines, sending 51 bytes.

There is no RCLK/Strobe like data in the bus. Only SCLK, SDATA1, SDATA2 signals.

Rising edge used to capture bit. No Time gap between bytes.

Data is sending every seconds

Every data is start at 0x12 in Data1 line while Data2 line data is 0xFF
That should used to capture the start point of the data stream

After start, first 12 bytes are the 12digits in the display in the same format as 7Segments including the decimal point

The decoding key table is as below, this could be any character which can be shown in 7 segments

7 Segment Map is below.

/*      ----------------

/ \      A       / \

/   \------------/   \

|   |            |   |

| F |            | B |

\   /------------\   /

\ /      G       \ /

/ \--------------/ \

/   \            /   \

|   |            |   |

| E |            | C |

\   /------------\   /     _

\ /      D       \ /    / H \

-----------------      \ _ /

bits 7, 6, 5, 4, 3, 2, 1, 0

A, B, C, D, E, F, G, H

*/

Below shows the extracted data from captured data